Isolated vRouter Setup and Testing with Trex

Test Methodology

6.20.2019 (1.5)

| Version | Date | Author(s) | Comment |
| --- | --- | --- | --- |
| 1.0 | 8.20.2018 | Joseph Gasparakis joseph.gasparakis@intel.com Subarna Kar subarna.kar@intel.com Savannah Loberger savannah.loberger@intel.com Wang, Yipeng yipeng1.wang@intel.com | Initial Version |
| 1.1 | 2.4.2019 | Savannah Loberger savannah.loberger@intel.com Wang, Yipeng yipeng1.wang@intel.com | Added updates with commit IDs and information for DPDK setup required before running vRouter. Update to using vRouter 5.0 version. |
| 1.2 | 7.2.2019 | Matthew Davis Matthew.Davis.2@team.telstra.com | Typo fixes Additional commands (wget, tar etc) sandeshy.hh workaround added |
| 1.3 | 2.19.2019 | Joseph Gasparakis joseph.gasparakis@intel.com Savannah Loberger savannah.loberger@intel.com | Added testing, tools information, and appendix sections |
| 1.4 | 6.20.2019 | Joseph Gasparakis joseph.gasparakis@intel.com Savannah Loberger savannah.loberger@intel.com | More performance related analysis info added |
| 1.5 | 6.21.2019 | Matthew Davis Matthew.Davis.2@team.telstra.com | Updated versions for trex, dpdk, qemu. Added apt dependencies Several other notes added |

Table of Contents

Overview	4

Goals	4

Setup Information	4

Installation	5

Machine 1 (vRouter and Qemu)	5

Install vRouter	5

Install Qemu	7

Machine 2 (Trex and DPDK)	7

Install DPDK	7

Install Trex	8

Running Isolated vRouter	8

Overview	8

Start vRouter	9

Setup DPDK	9

Run vRouter	10

Setup vRouter	10

Add Interfaces to vRouter	10

Using vTest	12

Example xml File	12

Run vTest	15

Start and Setup VM	15

Prep VM	15

Start VM	16

Configure VM	16

Install L2FWD in VM	17

Setup Trex to send MPLS/UDP Packets	17

DPDK Nic Setup	17

Trex configuration	18

Sample Packet File for Trex	19

Running Traffic	20

Forwarding Packet(s) in VM	20

Running L2FWD	20

Running Trex	20

Trex Stateless Mode	20

Performance Analysis	20

vRouter  Throughput	20

74B inner packet size (total 122B bytes encapsulated packet)	20

200B inner packet size (total 248B bytes encapsulated packet)	21

HTOP Overview 74B Sized Packets:	22

HTOP Overview 200B Sized Packets:	22

Hotspots per core for 74B Sized Packets	23

Hotspots per core for 200B Sized Packets	27

PCM data for 74B Sized Packets	31

PCM data for 200B Sized Packets	32

Conclusions	33

References	34

Appendix	34

mpls_udp_800_flows_sudp_122B.py	34

mpls_udp_800_flows_sudp_248B.py	35

mpls_add_original_800.xml	36

Legal	40

# Overview

## Goals

- This setup was originally created in order to find the limitations/bottlenecks of vRouter version 4.0 to then implement optimizations of the virtual router (vRouter) that is used in Tungsten Fabric (tungstenfabric.io).

- Isolated vRouter is the vRouter running outside of the usual Contrail environment. There is no agent or other software controlling the flows or paths. Programming of the datapath of the vRouter is done by the user through vrouter utilities such as vtest tool – the vrouter unit test framework, vif – vrouter interface tool, etc.

- The eventual goal is to run isolated vRouter from the master branch. That would provide a good opportunity for posting patches to the community and continue to improve Tungsten Fabric.

Machine 2

Machine 1

## Setup Information

- Ubuntu 16.04 LTS

- X710-DA4 NICs

  - 2 ports connected on each machine

# Installation

## Machine 1 (vRouter and Qemu)

### Install vRouter

- As root, Download the code:

# cd /root

# git clone https://github.com/Juniper/contrail-vrouter.git vrouter

# git clone https://github.com/Juniper/contrail-build.git tools/build

# git clone https://github.com/Juniper/contrail-sandesh.git tools/sandesh

# git clone https://github.com/Juniper/contrail-dpdk.git third_party/dpdk

# git clone https://github.com/Juniper/contrail-common/ src/contrail-common

Now, the directory tree should look like below:

root

|

q---vrouter (this is the contrail-vrouter)

|

---tools

|          |

|          ----build (this is the contrail-build)

|          |

|          ----sandesh (this is should be the contrail-sandesh)

|

---third_party

|          |

|          ----dpdk (this should be the contrail-dpdk)

|

---src

|          |

|          ----contrail-common (this is another library needed for sandesh to work)

|

---SConstruct (this should be the file you copy from contrail-build)

- Enter all the folders and checkout the following commit-ids. This is ensure that all the project is on the same page and to work together successfully.

# cd <folder>

# git checkout <commit-id>

| Folder | Commit-id |
| --- | --- |
| vrouter | bdf961e447ecada259548905e1582a8696878443 |
| third_party/dpdk | c5841c5284bca2f6f1afe077131489674324db1c |
| tools/sandesh | b5d5c1ee1117f59d8f00de620c8b9db236f6cb1e |
| tools/build | a99cefda8c8b22174347e276ad97b85155086874 |
| src/contrail-common | c93ef4b32cb64faac8267e2b7cd58c5c1ecb4f87 |

- Copy the SConstruct file to the root folder:

# cd /root

# cp tools/build/SConstruct ./

- To compile you may need the following packages (you may need to install each package individually as we have found that the package manager may skip some):

# apt get install -y libboost-all-dev libnl-genl-3-dev libxml2-dev liburcu-dev byacc flex libpcap-dev scons python python-pip pkg-config zlib1g-dev libglib2.0-dev libfdt-dev libpixman-1-dev cloud-image-utils bison binfmt-support

For the setup, the following packages were also needed

# wget http://ubuntu-cloud.archive.canonical.com/ubuntu/pool/main/libu/liburcu/liburcu2_0.8.5-1ubuntu1~cloud0_amd64.deb

# wget http://ubuntu-cloud.archive.canonical.com/ubuntu/pool/main/libu/liburcu/liburcu-dev_0.8.5-1ubuntu1~cloud0_amd64.deb

The package name may vary. The build system should warn you if you lack some packages.

To install these packages run

# sudo dpkg –i liburcu-dev_0.8.5-1ubuntu1~cloud0_amd64.deb

# sudo dpkg –i liburcu2_0.8.5-1ubuntu1~cloud0_amd64.deb

You may also have to also install the following .deb files (wget then dpkg -i):

- https://master.dl.sourceforge.net/project/libipfix/RELEASES/libipfix-dev_0.8.1-1ubuntu1_amd64.deb

- http://downloads.datastax.com/cpp-driver/ubuntu/16.04/cassandra/v2.11.0/cassandra-cpp-driver_2.11.0-1_amd64.deb

- http://downloads.datastax.com/cpp-driver/ubuntu/16.04/cassandra/v2.11.0/cassandra-cpp-driver-dev_2.11.0-1_amd64.deb

- http://downloads.datastax.com/cpp-driver/ubuntu/16.04/cassandra/v2.11.0/cassandra-cpp-driver-dbg_2.11.0-1_amd64.deb

- Compile vrouter, if no option is selected the default build is debug, however the production has better performance: (-jn may not work, if error, remove the -j option)

# cd /root

# scons vrouter -j2  --opt=production/debug/...

If you get an error about sandeshy.hh, try running:

# grep -r sandeshy\.hh -l | xargs sed -i 's/sandeshy\.hh/sandeshy\.h/g'

- After a successful build the DPDK-Vrouter binary is located at:

/root/build/debug/vrouter/dpdk/contrail-vrouter-dpdk

Or

/root/build/production/vrouter/dpdk/contrail-vrouter-dpdk

### Install Qemu

- In root directory, download qemu source code and install (we used qemu-2.11.1 in our setup)

- Download the zip from this site: https://github.com/qemu/qemu/tree/stable-2.5

# wget https://github.com/qemu/qemu/archive/v2.11.1.tar.gz

# tar -xzf v2.11.1.tar.gz

# cd qemu-2.11.1

# ./configure

# make

- The build for qemu can take a couple hours so better use the –j option of make (see your distro’s man pages), depending the number of cores in your system

## Machine 2 (Trex and DPDK)

### Install DPDK

- We used version 18.05.1, you can find all dpdk source code from www.dpdk.org

$ wget https://fast.dpdk.org/rel/dpdk-18.05.1.tar.xz

$ tar xf dpdk-18.05.1.tar.xz

$ cd dpdk-18.05.1

- Build the DPDK environment (x86_64-native-linuxapp-gcc).

$ sudo apt install libnuma-dev

$ make config T=x86_64-native-linuxapp-gcc

$ make # use –j

- Become superuser (if not already)

# sudo su

- Insert the igb_uio module

# modprobe uio

# insmod ./build/kmod/igb_uio.ko

Setup hugepages and enter the amount of hugepages you want (64 is a good number to start with)

# mkdir -p /mnt/huge

# mount -t hugetlbfs nodev /mnt/huge

# echo 64 > /sys/devices/system/node/node0/hugepages/hugepages-2048kB/nr_hugepages

# # repeat for other nodes if you have them

# echo 64 > /sys/devices/system/node/node1/hugepages/hugepages-2048kB/nr_hugepages

### Install Trex

- The setup of DPDK, explained above, must be done before running Trex.

- Install a dependency

# sudo apt-get install python3-distutils

- We used Trex version 2.52

# mkdir trex

# cd trex

# wget --no-cache http://trex-tgn.cisco.com/trex/release/v2.52.tar.gz

# tar –xzf v2.52.tar.gz

# Running Isolated vRouter

## Overview

- Diagram of our configuration of vRouter:

VIF 0/0 is the bottom left one. VIF 0/1 is the bottom right.

Here are the MAC addresses for this setup.

| Interface | Description | MAC | Different for you |
| --- | --- | --- | --- |
| Eth0 | Inside the VM | 02:e9:ee:49:c3:bc | No. Use this MAC value |
| Eth1 | Inside the VM | 02:e9:ee:49:c3:bd | No. Use this MAC value |
| VIF 0/0 | Physical NIC | 3c:fd:fe:9c:5b:19 | Yes, lookup with `ip a` |
| VIF 0/1 | Physical NIC | 3c:fd:fe:9c:5b:18 | Yes, lookup with `ip a` |
| VIF 0/2 | vRouter interface, looking up into VM | 00:00:5e:00:01:00 | No. Use this MAC value |
| VIF 0/3 | vRouter interface, looking up into VM | 00:00:5e:00:01:?? | No. Use this MAC value |

## Start vRouter

### Setup DPDK

Return to the first machine. Before running vRouter, setup hugepages and insert the igb_uio modules. The following commands have to be ran before running vRouter after a reboot of the machine.

- Setup hugepages. 12 pages of 1G and 1024 of 2M seems to be a good amount. There are a few ways to do this. The easiest is to change the grub command line. (This way they survive reboot.)

  - Edit /etc/default/grub

  - Append the following to GRUB_CMDLINE_LINUX

  - default_hugepagesz=1G hugepagesz=1G hugepages=12 hugepagesz=2M hugepages=1024

  - Run sudo update-grub

  - Reboot

- For Ubuntu, the vRouter references the hugepage information from the “/proc/mount” file. Verify that the entry was made in the file.

# grep pagesize=1G /proc/mounts

- Insert igb_uio module.

# modprobe uio

# cd /root

# insmod build/production/vrouter/dpdk/x86_64-native-linuxapp-gcc/kmod/igb_uio.ko

- Choose the 2 physical interfaces you want to use. Write down their MAC address, PCI address and linux interface name.

# ./third_party/dpdk/usertools/dpdk-devbind.py --status # for pci and linux interface name

# ip a # to get mac

- Bind devices to igb_uio. Replace the <interfaces> with your devices names.

# ifconfig <eth device> down

# ./third_party/dpdk/usertools/dpdk-devbind.py --bind=igb_uio <pcie address of nic>

# ./third_party/dpdk/usertools/dpdk-devbind.py --bind=igb_uio <pcie address of nic>

### Run vRouter

- Command to start vRouter:

# taskset 0x3f ./build/production/vrouter/dpdk/contrail-vrouter-dpdk --no-daemon --socket-mem 1024,1024

(or perhaps debug instead of production)

The 0x3f string pins vrouter to specific cores. This helps with performance. (Other processes may be scheduled to these CPUs. You can get a tiny bit more performance by excluding those other processes by adding the isolcpus=x,y,z argument in your grub commandline.)

## Setup vRouter

Setup config file

sudo mkdir –p /etc/contrail

sudo vim /etc/contrail/contrail-vrouter-agent.conf

Add the following to the file

[DEFAULT]
platform=dpdk

### Add Interfaces to vRouter

You can add physical and virtual devices to the vRouter. You can run vRouter with only one interface of each. However, for this setup you need:

- 2 physical interfaces

- 2 virtual interfaces

- Add 2 physical interfaces to vRouter. The MAC addresses should match the ones for these interfaces which you wrote down earlier:

# ./build/production/vrouter/utils/vif --add 0 --mac 3c:fd:fe:9c:5b:19 --vrf 0 --type physical --pmd

# ./build/production/vrouter/utils/vif --add 1 --mac 3c:fd:fe:9c:5b:18 --vrf 0 --type physical --pmd

- Add 2 virtual interfaces to vRouter: 1

# ./build/production/vrouter/utils/vif --add 2 --mac 00:00:5e:00:01:00 --vrf 0 --type virtual --transport pmd --pmd --policy

# ./build/production/vrouter/utils/vif --add 3 --mac 00:00:5e:00:01:01 --vrf 0 --type virtual --transport pmd --pmd --policy

(“--policy” sets the policy flag to on, so that packets from the VM will go through the flow lookup.)

Check to see what has been set up.

# ./build/production/vrouter/utils/vif --list

It should look like this:

Vrouter Interface Table

Flags: P=Policy, X=Cross Connect, S=Service Chain, Mr=Receive Mirror

Mt=Transmit Mirror, Tc=Transmit Checksum Offload, L3=Layer 3, L2=Layer 2

D=DHCP, Vp=Vhost Physical, Pr=Promiscuous, Vnt=Native Vlan Tagged

Mnp=No MAC Proxy, Dpdk=DPDK PMD Interface, Rfl=Receive Filtering Offload, Mon=Interface is Monitored

Uuf=Unknown Unicast Flood, Vof=VLAN insert/strip offload, Df=Drop New Flows, L=MAC Learning Enabled

Proxy=MAC Requests Proxied Always, Er=Etree Root, Mn=Mirror without Vlan Tag, Ig=Igmp Trap Enabled

vif0/0      PMD: 0 (Speed 10000, Duplex 1)

Type:Physical HWaddr:f8:f2:1e:05:ff:a4 IPaddr:0.0.0.0

Vrf:0 Mcast Vrf:65535 Flags:TcL3L2Dpdk QOS:0 Ref:15

RX device packets:103  bytes:33470 errors:0

RX port   packets:101 errors:0

RX queue errors to lcore 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

RX packets:101  bytes:33330 errors:101

TX packets:0  bytes:0 errors:0

Drops:101

vif0/1      PMD: 1 (Speed 10000, Duplex 1)

Type:Physical HWaddr:f8:f2:1e:05:ff:a6 IPaddr:0.0.0.0

Vrf:0 Mcast Vrf:65535 Flags:TcL3L2Dpdk QOS:0 Ref:15

RX device packets:102  bytes:33140 errors:0

RX port   packets:90 errors:0

RX queue errors to lcore 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

RX packets:90  bytes:29700 errors:90

TX packets:0  bytes:0 errors:0

Drops:90

vif0/2      PMD: 2

Type:Virtual HWaddr:00:00:5e:00:01:00 IPaddr:0.0.0.0

Vrf:0 Mcast Vrf:65535 Flags:PL3L2Dpdk QOS:0 Ref:15

RX queue errors to lcore 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

RX packets:0  bytes:0 errors:0

TX packets:0  bytes:0 errors:0

vif0/3      PMD: 3

Type:Virtual HWaddr:00:00:5e:00:01:00 IPaddr:0.0.0.0

Vrf:0 Mcast Vrf:65535 Flags:PL3L2Dpdk QOS:0 Ref:15

RX queue errors to lcore 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

RX packets:0  bytes:0 errors:0

TX packets:0  bytes:0 errors:0

Drops:0

### Using vTest

- vTest is the vrouter test tool that allows you to control the vRouter through xml file. Using vTest we are able to run vRouter without the agent present.

### Example xml File

For our setup we created 2 nexthops, 1 route lookup, 1 flow request, and added an MPLS label to the VM.

Some of the MAC addresses in the following file must be changed. Read the XML comments.

Contents of mpls_add.xml:

<?xml version="1.0"?> <test> <test_name>Adds nexthop, </test_name> <message> <vr_nexthop_req> <!--This nexthop is to receive the tunneled packet and is obtained by route lookup of outer dst IP --> <h_op>Add</h_op> <nhr_type>1</nhr_type> <!-- RCV type --> <nhr_id>21</nhr_id> <!-- you can choose any number, other than 0 which is the default nexthop to drop the pkt --> <nhr_family>2</nhr_family> <!-- type AF_INET --> <nhr_encap_oif_id>1</nhr_encap_oif_id> <!-- In case of a non tunneled packet, it will be sent out of the other NIC --> <nhr_vrf>0</nhr_vrf> <nhr_flags>1</nhr_flags> </vr_nexthop_req> <return>0</return> </message> <message> <vr_nexthop_req> <!--This nexthop is for inner packet, obtained from mpls tag and points to the corresponding VM the pkt should be sent to --> <h_op>Add</h_op> <nhr_type>2</nhr_type> <!-- encap type --> <nhr_id>20</nhr_id> <nhr_family>7</nhr_family> <!-- type AF_BRIDGE --> <nhr_encap_oif_id>2</nhr_encap_oif_id> <!--pointing to the VM which is interface 2 in our setup --> <nhr_encap>02:e9:ee:49:c3:bc:00:00:5e:00:01:00:08:00</nhr_encap> <!--MAC address of interface in VM and MAC address of virtual interface in vrouter, then 08:00 --> <nhr_vrf>0</nhr_vrf> <nhr_flags>7</nhr_flags> </vr_nexthop_req> <return>0</return> </message> <message> <vr_nexthop_req> <!--This nh is for pkts coming from 3rd vif from VM to go out of the 1st physical vif to go to trex --> <h_op>Add</h_op> <nhr_type>3</nhr_type> <!-- Tunnel type --> <nhr_id>22</nhr_id> <nhr_encap_oif_id>1</nhr_encap_oif_id> <!--go out of NIC 1 --> <nhr_encap>68:05:ca:04:4a:91:3c:fd:fe:9c:5b:18:08:00</nhr_encap> <nhr_tun_sip>33686019</nhr_tun_sip> <!-- 2.2.2.3 --> <nhr_tun_dip>33686021</nhr_tun_dip> <!-- 2.2.2.5 --> <nhr_tun_sport>0</nhr_tun_sport> <nhr_tun_dport>0</nhr_tun_dport> <nhr_vrf>0</nhr_vrf> <nhr_flags>65</nhr_flags> <!--NH_FLAG_TUNNEL_UDP_MPLS, NH_FLAG_VALID --> </vr_nexthop_req> <return>0</return> </message> <message> <vr_route_req> <!--This route lookup is to connect pkts with outer dst IP 2.2.2.2 to RCV NH of id 21. In full setup, the IP shd be same as vhost0's IP --> <h_op>Add</h_op> <rtr_family>2</rtr_family> <rtr_nh_id>21</rtr_nh_id> <!--ID of the RCV nexthop defined earlier --> <rtr_prefix>2.2.2.2</rtr_prefix> <!--this can be any IP address you mention in outer dst IP in TREX --> <rtr_prefix_len>8</rtr_prefix_len> <rtr_vrf_id>0</rtr_vrf_id> </vr_route_req> <return>0</return> </message> <message> <vr_route_req> <!--This is to create bridge entry for pkts coming out of the VM to reach trex port through NIC --> <!--Naturally it shd search for TREx dst MAC/RX VM's MAC in bridge, in this case, l2fwd, no mac updating, so pkt has same dst MAC as being sent by TREX, as set in udp_2pkt_simple.py --> <h_op>Add</h_op> <rtr_family>7</rtr_family> <rtr_nh_id>22</rtr_nh_id> <rtr_label_flags>3</rtr_label_flags> <!-- BR_BE_VALID_FLAG --> <rtr_mac>02:e9:ee:49:c3:bc</rtr_mac> <rtr_label>5</rtr_label> <!-- does this become MPLS label? --> <rtr_vrf_id>0</rtr_vrf_id> </vr_route_req> <return>0</return> </message> <message> <vr_flow_req> <!--This flow is for the inner packet lookup after mpls header decapsulation, it looks at the 5 tuples --> <fr_op>flow_set</fr_op> <fr_flow_sip_l>50397441</fr_flow_sip_l> <!--decimal version of src IP 3.1.1.1 --> <fr_flow_sip_u>0</fr_flow_sip_u> <fr_flow_dip_l>67174657</fr_flow_dip_l> <!-- decimal version 4.1.1.1 --> <fr_flow_dip_u>0</fr_flow_dip_u> <fr_family>2</fr_family> <fr_index>-1</fr_index> <fr_flags>1</fr_flags> <fr_flow_proto>17</fr_flow_proto> <!-- UDP --> <fr_flow_sport>60185</fr_flow_sport><!-- this port is needed for it to know this is MPLS pkt, this value is hardcoded in vrouter code --> <fr_flow_nh_id>0</fr_flow_nh_id> <fr_action>2</fr_action> <!-- FORWARD --> <fr_flow_dport>60185</fr_flow_dport> </vr_flow_req> <return>0</return> </message> <message> <vr_mpls_req> <!--Adding mpls tag, which should be done by agent when VM comes up --> <h_op>Add</h_op> <mr_label>4</mr_label> <!--Randomly chosen, shd be mentioned in TREX pkt --> <mr_nhid>20</mr_nhid> <!--points to the ENCAP nexthop defined earlier --> </vr_mpls_req> <return>0</return> </message> <message> <vr_mpls_req> <h_op>Add</h_op> <mr_label>5</mr_label> <mr_nhid>21</mr_nhid> </vr_mpls_req> <return>0</return> </message> </test>

### Run vTest

- Add mpls tags, next hops, and routes to vRouter, create the new file and store mpls_add.xml at /root :

# cd /root

# ./build/production/vrouter/utils/vtest/vtest mpls_add.xml

You can inspect the hops and flows with:

# ./build/production/vrouter/utils/nh --list

#./build/production/vrouter/utils/flow -l

## Start and Setup VM

### Prep VM

- Download and prep images (Ubuntu 16)

IMG_DATA="xenial-server-cloudimg-amd64-disk1.img"

IMG_UEFI="xenial-server-cloudimg-amd64-uefi1.img"

URL_PREFIX=https://cloud-images.ubuntu.com/xenial/current/

USER_DATA="user-data.img"

wget ${URL_PREFIX}${IMG_DATA} --show-progress

wget ${URL_PREFIX}${IMG_UEFI} --show-progress

/root/qemu-2.11.1/qemu-img resize "${IMG_DATA}" +128G

- Prep user data for cloud init. Edit file user-data.txt

#cloud-config

password: myPass

chpasswd: { expire: False }

ssh_pwauth: True

- Create user data image

cloud-localds "$USER_DATA" user-data.txt

### Start VM

- Qemu command to start VM (using Qemu built from source),

“path” is to the socket that was created after adding virtual interfaces to the

isolated vrouter:

# taskset 0xf00 ./qemu-2.11.1/x86_64-softmmu/qemu-system-x86_64 \

-m 3G \

-drive "file=${IMG_UEFI},format=qcow2" \

-drive "file=${IMG_DATA},format=qcow2" \

-drive "file=${USER_DATA},format=raw" \

-cpu host \

-object memory-backend-file,id=mem,size=3072M,mem-path=/mnt/huge,share=on \

-numa node,memdev=mem \

-mem-prealloc \

-mem-path /mnt/huge,prealloc=on,share=on \

-smp cores=4,threads=1,sockets=1 \

--enable-kvm \

-chardev socket,id=chr0,path=/var/run/vrouter/uvh_vif_2 \

-netdev type=vhost-user,id=net0,chardev=chr0 \

-device virtio-net-pci,netdev=net0,mac=02:e9:ee:49:c3:bc \

-chardev socket,id=chr1,path=/var/run/vrouter/uvh_vif_3 \

-netdev type=vhost-user,id=net1,chardev=chr1 \

-device virtio-net-pci,netdev=net1,mac=02:e9:ee:49:c3:bd  \

-nographic \

-device virtio-net-pci,netdev=net2 \

-netdev user,id=net2 \

;

If you’re doing this over a ssh connection (without a graphic window) you may need to remove the vnc argument and add

-serial mon:stdio

The login is user:ubuntu password: myPass

To quit qemu, use Ctrl-A X

You will be able to run `apt update` from inside the VM, but not ping, because the internet interface blocks icmp (and inbound ssh).

### Configure VM

- Open VM and stop network manager

# service network-manager stop

- Grow the root partition because the original cloud image has no room

# apt-get install cloud-initramfs-growroot

# reboot

### Install L2FWD in VM

- Inside the same VM, install and compile DPDK to use L2FWD to forward the incoming packets. Below are the list of commands that we ran to setup DPDK for L2FWD

# wget https://fast.dpdk.org/rel/dpdk-18.08.tar.xz

# tar xf dpdk-18.08.tar.xz

# cd dpdk-18.08

First build the DPDK environment (x86_64-native-linuxapp-gcc).

# apt install build-essential libnuma-dev python pkg-config

# make config T=x86_64-native-linuxapp-gcc

# make

Second insert the igb_uio module

# sudo modprobe uio

# sudo insmod build/kmod/igb_uio.ko

Setup hugepages and enter the amount of hugepages you want (64 is a good number to start with)

# mkdir -p /mnt/huge

# mount -t hugetlbfs nodev /mnt/huge

# echo 64 > /sys/devices/system/node/node0/hugepages/hugepages-2048kB/nr_hugepages

- Connect the two eth interfaces to DPDK

# ./usertools/dpdk-devbind.py –bind=igb_uio eth0

# ./usertools/dpdk-devbind.py –bind=igb_uio eth1

- Build L2FWD (RTE_SDK is the path to the DPDK folder)

# make -C examples RTE_SDK=$(pwd) RTE_TARGET=build O=$(pwd)/build/examples

## Setup Trex to send MPLS/UDP Packets

### DPDK Nic Setup

- Write down the interface name, MAC and PCI addresses of your interfaces (you won’t be able to use the normal tools later, after we change the bindings).

# cd trex/v2.53

# ./dpdk_nic_bind.py --status

# ip a

- Bind your nic to igb_uio, replace port numbers with your own

Or, you can use ./dpdk_nic_bind.py –t to display a table of information about your NICs

Using the displayed information, bind a minimum of 2 ports to igb_uio driver

# ./dpdk_nic_bind.py -b igb_uio 08:00.0 08:00.1

### Trex configuration

- Sample Trex Configuration File, stored at /etc/trex_cfg.yaml, add in your devices MAC addresses and port information

Contents of trex_cfg.yaml:

- port_limit : 2 # this option can limit the number of port of the platform version : 2 interfaces : ["08:00.0","08:00.1"] # the interfaces using ./dpdk_setup_ports.py -s port_info : # set the mac addr - dest_mac : [0xFF,0xFF,0xFF,0xFF,0xFF,0x18] # taken from router src_mac : [0xFF,0xFF,0xFF,0xFF,0xFF,0x91] # taken from ifconfig - dest_mac : [0xFF,0xFD,0xFF,0xFF,0xFF,0x19] # taken from router src_mac : [0xFF,0xFF,0xFF,0xFF,0xFF,0x90] # taken from ifconfig

### Sample Packet File for Trex

- Based on RFC7510 for the vRouter to accept the MPLSoUDP packets the UDP destination and source ports must be set to 6635

- Also, there must be time-to-live (TTL) set to a high enough value (ex. we used 63) otherwise the packet is dropped

Contents of mpls_udp_1pkt_simple.py:

from trex_stl_lib.api import * from scapy.contrib.mpls import * class STLS1(object): def create_stream (self): return STLStream( packet = STLPktBuilder( pkt = Ether()/IP(src="2.2.2.1",dst="2.2.2.2")/UDP(dport=6635,sport=6635)/MPLS(label=0x04,ttl=63)/Ether(dst="02:e9:ee:49:c3:bc")/IP(src="1.1.1.3",dst="1.1.1.4")/UDP(dport=6635,sport=6635)/(10*'x') ), mode = STLTXCont()) def get_streams (self, direction = 0, **kwargs): # create 1 stream return [ self.create_stream() ] # dynamic load - used for trex console or simulator def register(): return STLS1()

# Running Traffic

## Forwarding Packet(s) in VM

### Running L2FWD

- Forward the incoming traffic inside the VM on eth0 to eth1

# ./examples/l2fwd/build/app/l2fwd -l 0-3 -n 4 -- -p 0x03 --no-mac-updating

## Running Trex

### Trex Stateless Mode

- In one terminal of Machine 2 start trex in interactive mode (at this point there should be available hugepages for trex to run that was setup in a previous step with DPDK)

# cd trex/v2.52

# ./t-rex-64 -i

- In another terminal start the console for trex and start sending traffic, below configures the receiving port (port 1) to promiscuous mode to receive the incoming packets and sends MPLSoUDP packets from port 0 for 5 seconds

# cd trex/v2.52

# ./trex-console

trex> stats

trex> port_attr --port 1 --prom

trex> start -p 0 -d 5  -f stl/mpls_udp_1pkt_simple.py -m 100%

trex> stats

trex> quit

The “-m 100%” argument means “scale up traffic to 100% of line rate”.

# Performance Analysis

## vRouter  Throughput

All tests are done with 800 flows, varying outer source UDP port and inner IP source IP. Measurements are taken with the traffic running.

### 74B inner packet size (total 122B bytes encapsulated packet)

taskset 0x3fc00 ./build/production/vrouter/dpdk/contrail-vrouter-dpdk --no-daemon --socket-mem 1024,0

Trex file:  stl/production/testing/mpls_udp_800_flows_sudp_122B.py    XML file: mpls_add_original_800.xml

#### Throughput

|  | Tput (Gbps) |
| --- | --- |
| 122B packets | 3.97Gbps |

### 200B inner packet size (total 248B bytes encapsulated packet)

taskset 0x3fc00 ./build/production/vrouter/dpdk/contrail-vrouter-dpdk --no-daemon --socket-mem 1024,0

Trex file:  stl/production/testing/mpls_udp_800_flows_sudp_248B.py    XML file: mpls_add_original_800.xml

#### Throughput

|  | Tput (Gbps) |
| --- | --- |
| 248B packets | 7.85Gbps |

## HTOP Overview 74B Sized Packets:

Htop command that displays the per core utilization of the machine.

# htop

|  | 74B |
| --- | --- |
| Cores 10-17 = vRouter Cores 4-6 = Qemu/VM |  |

## HTOP Overview 200B Sized Packets:

|  | 200B |
| --- | --- |
| Cores 10-17 = vRouter Cores 4-6 = Qemu/VM |  |

## Hotspots per core for 74B Sized Packets

Perf command to collect highest cpu utilizing functions, or hotspots, per core. Replace <core number> with the core you want to view.

# perf top –C10

# perf top -C<core number>

| Core | 74B Packets |
| --- | --- |
| Core 10 (Forwarding/IO/Service) |  |
| Core 11 |  |
| Core 12 |  |
| Core 13 |  |
| Core 14 |  |
| Core 15 |  |
| Core 16 |  |
| Core 17 |  |

## Hotspots per core for 200B Sized Packets

| Core | 200B Packets |
| --- | --- |
| Core 10 (Forwarding/IO/Service) |  |
| Core 11 |  |
| Core 12 |  |
| Core 13 |  |
| Core 14 |  |
| Core 15 |  |
| Core 16 |  |
| Core 17 |  |

## PCM data for 74B Sized Packets

PCM tools can be downloaded from this link: https://github.com/opcm/pcm

The pcm tools will notify if there are missing dependencies or drivers keeping the run from being successful.

To install the tools on Linux, type 'make'. You will get all the utilities (pcm.x, pcm-memory.x, etc) built in the main PCM directory.

# git clone https://github.com/opcm/pcm.git

# cd pcm

# make

# modprobe msr ### only do it once, after each reboot

| PCM Tool | 74B |
| --- | --- |
| Pcm-pcie |  |
| Pcm-numa |  |
| Pcm-memory |  |

## PCM data for 200B Sized Packets

| PCM Tool | 200B |
| --- | --- |
| Pcm-pcie |  |
| Pcm-numa |  |
| Pcm-memory |  |

# Conclusions

In brief these are the proposed optimization targets but deeper analysis might be needed:

- All functions with high CPU count (mostly lookup functions)

  - vr_htable_find_hentry(): Initial patches from Intel (Yipeng Wang) https://review.opencontrail.org/#/c/51201/

  - vr_flow_lookup(): Needs to be optimized

  - URCU: There is a userspace RCU (provided by liburcu) used which potentially can be problematic if called in data path. Further investigation is needed.

- vHost from DPDK.org – Multiple benefits as the current vRouter vHost is lacking functionality such as:

  - live migration

  - running vRouter inside a container

  - potential performance degradation

- Packet re-disrtribution/NIC modeling – vr_dpdk_ethdev_rx_emulate()might be need to be written in a way to avoid multiple repeated for loops and consolidate them to one.

# References

- For more information about setting up and running DPDK, refer here:

https://doc.dpdk.org/guides/index.html

- For more information about trex, refer here:

https://trex-tgn.cisco.com/trex/doc/trex_manual.html#_download_and_installation

- For more information about running the trex console refer here:

https://trex-tgn.cisco.com/trex/doc/trex_console.html

- For more information about the vRouter tool, vif:

https://www.juniper.net/documentation/en_US/contrail2.0/topics/task/configuration/vrouter-cli-utilities-vnc.html

- For more information about the vRouter tool, vtest:

https://github.com/Juniper/contrail-vrouter/tree/master/utils/vtest

# Appendix

## mpls_udp_800_flows_sudp_122B.py

These are the contents of the file for trex that is used to generate 122B total sized packets with varying outer UDP source ports and varying inner source IP.

from trex_stl_lib.api import * from scapy.contrib.mpls import * class STLS1(object): def create_stream (self): #create base pkt base_pkt = Ether(dst="3c:fd:fe:9c:5b:78")/IP(src=="2.2.2.1",dst="2.2.2.2")/UDP(dport=6635,sport=6635)/MPLS(label=0x04,ttl=63)/Ether(dst="00:00:00:00:c3:bc")/IP(src="1.1.1.5",dst="1.1.1.4")/UDP(dport=6615,sport=6615) pad = 30 * 'x' vm = STLScVmRaw( [ STLVmFlowVar("src_port", min_value=6635, max_value=7435, size=2, op="inc"), STLVmFlowVar ( "ip_src", min_value="1.1.1.5", max_value="1.1.4.33", size=4, step=1,op="inc"), STLVmWrFlowVar (fv_name="ip_src", pkt_offset= "IP:1.src" ), # write ip to internal packet IP.src STLVmFixIpv4(offset = "IP"), # fix checksum STLVmWrFlowVar(fv_name="src_port", pkt_offset= "UDP.sport") # fix udp len ] ); return STLStream( packet = STLPktBuilder(pkt = base_pkt/pad, vm = vm),mode = STLTXCont()) def get_streams (self, direction = 0, flows=800, **kwargs): # create 1 stream return [ self.create_stream() ] # dynamic load - used for trex console or simulator def register(): return STLS1()

## mpls_udp_800_flows_sudp_248B.py

- This is the contents of the file for trex that is used to generate 248B total sized packets with varying outer UDP source ports and varying inner source IP.

from trex_stl_lib.api import * from scapy.contrib.mpls import * class STLS1(object): def create_stream (self): #create base pkt base_pkt = Ether(dst="3c:fd:fe:9c:5b:78")/IP(src="2.2.2.1",dst="2.2.2.2")/UDP(dport=6635,sport=6635)/MPLS(label=0x04,ttl=63)/Ether(dst="00:00:00:00:c3:bc")/IP(src="1.1.1.5",dst="1.1.1.4")/UDP(dport=6615,sport=6615) pad = 156 * 'x' vm = STLScVmRaw( [ STLVmFlowVar("src_port", min_value=6635, max_value=7435, size=2, op="inc"), STLVmFlowVar ( "ip_src", min_value="1.1.1.5", max_value="1.1.4.33", size=4, step=1,op="inc"), STLVmWrFlowVar (fv_name="ip_src", pkt_offset= "IP:1.src" ), # write ip to internal packet IP.src STLVmFixIpv4(offset = "IP"), # fix checksum STLVmWrFlowVar(fv_name="src_port", pkt_offset= "UDP.sport") # fix udp len ] ); return STLStream( packet = STLPktBuilder(pkt = base_pkt/pad, vm = vm),mode = STLTXCont()) def get_streams (self, direction = 0, flows=800, **kwargs): # create 1 stream return [ self.create_stream() ] # dynamic load - used for trex console or simulator def register(): return STLS1()

## mpls_add_original_800.xml

- This is the XML file used by vtest to program nexthops and flows to the vrouter. This XML matches with the trex traffic shared above and is used for the vRouter.

<?xml version="1.0"?> <test> <test_name>Adds nexthop, </test_name> <message> <vr_nexthop_req> <!--This nexthop is to receive the tunneled packet and is obtained by route lookup of outer dst IP --> <h_op>Add</h_op> <nhr_type>1</nhr_type> <!-- RCV type --> <nhr_id>21</nhr_id> <!-- you can choose any number, other than 0 which is the default nexthop to drop the pkt --> <nhr_family>2</nhr_family> <!-- type AF_INET --> <nhr_encap_oif_id>1</nhr_encap_oif_id> <!-- In case of a non tunneled packet, it will be sent out of the other NIC --> <nhr_vrf>0</nhr_vrf> <nhr_flags>1</nhr_flags> </vr_nexthop_req> <return>0</return> </message> <message> <vr_nexthop_req> <!--This nexthop is for inner packet, obtained from mpls tag and points to the corresponding VM the pkt should be sent to --> <h_op>Add</h_op> <nhr_type>2</nhr_type> <!-- encap type --> <nhr_id> 20</nhr_id> <nhr_family>7</nhr_family> <!-- type AF_BRIDGE --> <nhr_encap_oif_id>2</nhr_encap_oif_id> <!--pointing to the VM which is interface 2 in our setup --> <nhr_encap>00:00:00:00:c3:bc:00:00:5e:00:01:00:08:00</nhr_encap> <!--MAC address of interface in VM and MAC address of virtual interface in vrouter, then 08:00 --> <nhr_vrf>0</nhr_vrf> <nhr_flags>7</nhr_flags> <!--8388615 for L2_CONTROL_DATA --> </vr_nexthop_req> <return>0</return> </message> <message> <vr_nexthop_req> <!--This nh is for pkts coming from 3rd vif from VM to go out of the 1st physical vif to go to trex --> <h_op>Add</h_op> <nhr_type>3</nhr_type> <!-- Tunnel type --> <nhr_id>22</nhr_id> <nhr_encap_oif_id>1</nhr_encap_oif_id> <!--go out of NIC 1 --> <nhr_encap>68:05:ca:04:4a:91:3c:fd:fe:9c:5b:18:08:00</nhr_encap> <nhr_tun_sip>33686019</nhr_tun_sip> <nhr_tun_dip>16908802</nhr_tun_dip> <nhr_tun_sport>0</nhr_tun_sport> <nhr_tun_dport>0</nhr_tun_dport> <nhr_vrf>0</nhr_vrf> <nhr_flags>65</nhr_flags> <!--NH_FLAG_TUNNEL_UDP_MPLS, NH_FLAG_VALID --> </vr_nexthop_req> <return>0</return> </message> <message> <vr_route_req> <!--This route lookup is to connect pkts with outer dst IP 2.2.2.2 to RCV NH of id 21. In full setup, the IP shd be same as vhost0's IP --> <h_op>Add</h_op> <rtr_family>2</rtr_family> <rtr_nh_id>21</rtr_nh_id> <!--ID of the RCV nexthop defined earlier --> <rtr_prefix>2.2.2.2</rtr_prefix> <!--this can be any IP address you mention in outer dst IP in TREX --> <rtr_prefix_len>8</rtr_prefix_len> <rtr_vrf_id>0</rtr_vrf_id> </vr_route_req> <return>0</return> </message> <message> <vr_route_req> <!--This is to create bridge entry for pkts coming out of the VM to reach trex port through NIC --> <!--Naturally it shd search for TREx dst MAC/RX VM's MAC in bridge, in this case, l2fwd, no mac updating, so pkt has same dst MAC as being sent by TREX, as set in udp_2pkt_simple.py --> <h_op>Add</h_op> <rtr_family>7</rtr_family> <rtr_nh_id>22</rtr_nh_id> <rtr_label_flags>3</rtr_label_flags> <!-- BR_BE_VALID_FLAG --> <rtr_mac>00:00:00:00:c3:bc</rtr_mac> <rtr_label>5</rtr_label> <!-- does this become MPLS label? --> <rtr_vrf_id>0</rtr_vrf_id> </vr_route_req> <return>0</return> </message> <message> <vr_mpls_req> <!--Adding mpls tag, which shd be done by agent when VM comes up --> <h_op>Add</h_op> <mr_label>4</mr_label> <!--Randomly chosen, shd be mentioned in TREX pkt --> <mr_nhid>20</mr_nhid> <!--points to the ENCAP nexthop defined earlier --> </vr_mpls_req> <return>0</return> </message> <message> <vr_mpls_req> <!-- Do we need to add this? Maybe it should point to the nexthop in RX machine??? --> <h_op>Add</h_op> <mr_label>5</mr_label> <mr_nhid>21</mr_nhid> </vr_mpls_req> <return>0</return> </message> <message> <vr_flow_req> <!--This flow is for the inner packet lookup after mpls header decapsulation, it looks at the 5 tuples --> <fr_op>flow_set</fr_op> <fr_flow_sip_l>83951873</fr_flow_sip_l> <!--decimal version of src IP, 1.1.1.5 --> <fr_flow_sip_u>0</fr_flow_sip_u> <fr_flow_dip_l>67174657</fr_flow_dip_l> <!-- 1.1.1.4 --> <fr_flow_dip_u>0</fr_flow_dip_u> <fr_family>2</fr_family> <fr_index>-1</fr_index> <fr_flags>1</fr_flags> <fr_flow_proto>17</fr_flow_proto> <!-- UDP --> <fr_flow_sport>55065</fr_flow_sport><!-- this port is needed for it to know this is MPLS pkt, this value is hardcoded in vrouter code --> <fr_flow_nh_id>0</fr_flow_nh_id> <!--change nh_id to 20 and add src_nh_id to 22 for offloads to work --> <fr_action>2</fr_action> <!-- FORWARD --> <fr_flow_dport>55065</fr_flow_dport> </vr_flow_req> <return>0</return> </message> <control> <flow_count>800</flow_count> </control> </test>

# Legal

No license (express or implied, by estoppel or otherwise) to any intellectual property rights is granted by this document.

This document contains information on products, services and/or processes in development.  All information provided here is subject to change without notice.

Contact your Intel representative to obtain the latest forecast, schedule, specifications and roadmaps.

The products and services described may contain defects or errors known as errata which may cause deviations from published specifications. Current characterized errata are available on request. No product or component can be absolutely secure.

You may not use or facilitate the use of this document in connection with any infringement or other legal analysis concerning Intel products described herein. You agree to grant Intel a non-exclusive, royalty-free license to any patent claim thereafter drafted which includes subject matter disclosed herein.

Forecasts:  Any forecasts of requirements for goods and services are provided for discussion purposes only. Intel will have no liability to make any purchase pursuant to forecasts.  Any cost or expense you incur to respond to requests for information or in reliance on any forecast will be at your own risk and expense.

Business Forecast: Statements in this document that refer to Intel’s plans and expectations for the quarter, the year, and the future, are forward-looking statements that involve a number of risks and uncertainties. A detailed discussion of the factors that could affect Intel’s results and plans is included in Intel’s SEC filings, including the annual report on Form 10-K.

Copies of documents which have an order number and are referenced in this document may be obtained by calling 1-800-548-4725 or by visiting  www.intel.com/design/literature.htm.

Intel, the Intel logo, [List the Intel trademarks in your document] are trademarks of Intel Corporation or its subsidiaries in the U.S. and/or other countries.

*Other names and brands may be claimed as the property of others

© Intel Corporation.